PMINFO.DOC

NAME

PMINFO - measures performance of protected mode and extended

memory.

SYNOPSIS

PMINFO

DESCRIPTION

PMINFO makes a variety of performance measurements concerning

protected mode and extended memory on your computer.

When you type PMINFO <RETURN> at the DOS prompt, PMINFO

displays a message about your computer similar to this:

Protected Mode and Extended Memory Performance Measurement -- 3.25

Copyright 1988 by Rational Systems, Inc.

DOS memory  Extended memory             CPU is 10.0 MHz 80286.

----------  ---------------

639              384   K bytes configured (according to BIOS).

640              384   K bytes physically present (SETUP).

395              384   K bytes available for DOS/16M programs.

(extended memory range 1024K to 1408K)

3.1 (1.0)        3.1 (1.0)   MB/sec word transfer rate (wait states).

Overall cpu and memory performance (non-floating point) for typical

DOS programs is 1.26 times an 8MHz IBM PC/AT.

Protected/Real switch rate = 3937/sec (253 usec/switch, 90 up +

163 down) using DOS/16M switch mode 2.

The top information line shows that the CPU is an Intel 80286

processor running at 10 MHz.  (All of the time-based

measurements made by PMINFO may vary slightly from run to run.)

Below are the configuration and timings made for both the low

memory ("DOS memory") and extended memory.  If your computer is

not equipped with extended memory, or none is available for

DOS/16M, the extended memory measurements may be omitted

("--").  There can be several variations in the memory

configuration.

The line "according to BIOS" shows the information provided by

the BIOS (interrupts 12h and 15h function 88h).  The line

"(SETUP)" is obtained directly from the CMOS RAM as set by the

computer's setup program.  It is displayed only if the numbers

are different from the BIOS line.  They will be different for

computers where the BIOS has reserved memory for itself (as is

the case on this IBM PS/2 in the example), or if another

program has allocated some memory and is intercepting the BIOS

configuration requests to report less memory available than is

physically configured.  The "extended memory range" show the

low and high addresses available to DOS/16M in extended memory.

One or more lines are displayed that show memory speed.  PMINFO

tries to determine the memory architecture.  Some architectures

perform well under some circumstances and poorly under

others:  PMINFO will show both the best and worst cases.  The

architectures detected are cache, interleaved, page-mode (or

static column), and direct.  Also, on 80386-based systems,

measurements are made using 32-bit accesses. These measurements

are reported as the number of megabytes per second that can be

transferred.  Also, the number of wait states is reported in

parentheses.  The wait states can be a fractional number, like

0.5, if there is a wait state on writes but not on reads.

Memory bandwidth (i.e., how fast the CPU can access memory)

accounts for 60% to 70% of the performance for typical programs

(that are not heavily dependent on floating point math).

A performance metric is displayed, showing the expected

throughput for the computer relative to a standard 8MHz IBM

PC/AT (disk accesses and floating point are excluded).

Finally, the speed that the computer can switch between real

and protected mode is displayed, both as the maximum number

of round-trip switches that can occur per second, and the

time of a switch, broken out into the real-to-protected (up)

and protected-to-real (down) components.