Discription :

This documentation only dicribe the operation of the supplied

disk.Please reference the user manual for detail function description.

The disk supplied include 4 utility files below.

1.PALPGM.EXE -- this is the main file to be executed

2.PALPGM.DAT -- this file save previous data while

PALPGM.EXE terminated

3.PALFORM ----- this is a subdirectory include the

blank form files of all programable

array logic type used by PALPGM.EXE

4.README.DOC--- all updated message not printed on manual

will be included in this file.Be sure to read

this file before executing PALPGM.EXE.

* do not edit the blank form file by yourself,or may be

something error happend.

* these files must be at the same disk,and also must be at the

same default disk driver

Program execution :

Insert the programer card supplied on any slot of the computer,

while computer power off.

Put this disk at the default disk driver,put your fuse map file

( JEDEC format ) on any disk driver, and key in command below.

A>PALPGM <CR>

The on screen manual will be appeared.and select desired operation

as you want to do.

HAVE A GOOD JOB !

NOTE : be sure to back up 1 copy of the source disk,and save

at a safty place

Symble on blank form :

don't care about the 1st line of blank form.

N :no fuse.

x :fuse exist.

Symble on fuse map buffer :

1 :blow or blown.

0 :no blow or not blown

N :don't care(because no fuse here)

NOTE :1.JEDEC format do not supply symble N .But the program

can find out where are no fuse positions.

2.While editing fuse map buffer,only 1 and 0 can be fed.

If you feed 1 or 0 at no fuse position(N),don't worry

about that.The program will automatically find out where

it is,while blowing.

3.position N will not be checked,blown,verified.

4.be sure to reload your fuse map from disk after changing

new pal type.

DESIGN GOAL          :

The popular type (produced by many manufacturer and also

same program procedure with deferent program voltage) is the goal

of this programmer card.

REVISION DESCRIPTION :

V1.0-V1.1   : Internal used only in design department.

V2.0        : 1st released to the end user.

V2.1        : Load fuse map function bug has been solved.

Bug appeared on early version while 1st digit of

check-sum equals 'F'(Default Symble).

V2.2        : Up to now PALPGM.EXE has no blown procedure on

polarity fuses and shared product term fuses.

The p type and s type will be deleted.May be updated

on new version.

Deleted type shown below

20 pin  : 16P8, 16RP8,16RP6,16RP4

24 pin  : 20S10,20RS4,20RS8,20RS10,20RA10

V2.3        : B-type device of MMI are added as follow.

MFG SYMBLE      DEVICE INCLUDED

MMI-A           MMI: standard,

-2(standard half power)

A(fast speed),

A-2(fast speed half power)

MMI-B           MMI: B(ultra fast speed),

Hardware Design Change Note for old owner:

1. R12   3.9K ---> 2.0K

2. VR4 adjustment change

2.1 select MFG to MMI-B,TYPE to 24 pin(any type)

2.2 read without any device inserted on TEXTOOL

2.3 use osciliscope to monitering pin 1

2.4 adj. VR4 for VIHH to 10.00 +/- 0.25 Volt while

reading

NOTE : the reading time may be very short for adj-

ustment.

VIHH (program)       VP (security)

MMI-A : 11.75 +/- 0.25 Volt  18.5 +/- 0.5 Volt

MMI-B : 10.00 +/- 0.25 Volt  18.5 +/- 0.5 Volt

TI    : 10.50 +/- 0.25 Volt  21.0 +/- 0.5 Volt

NS    : 11.75 +/- 0.25 Volt  18.5 +/- 0.5 Volt

############