BACKUP.ASM   USED TO BACKUP HARD DISKS

WRITTEN BY G. YOUNG, PO BOX 3218, NO. HOLLYWOOD, CA 91609

The  Winchester  hard disk offers  file storage approaching   mainframe

performance on microcomputers.  The problem  of backing up the  data is

a serious drawback, however.  The only option currently available is to

use  PIP to copy one file at a time.  To PIP an entire file  type would

quickly  fill a floppy  and not copy the rest of the files.  To PIP one

file at a time would be unbearably time consuming.  Another alternative

is obviously necessary.

BACKUP  is  used to  copy files  from  large  volume  hard disks to low

volume floppy disks for backup and recovery  purposes.   It is intended

for CP/M files where the entire file will fit on one floppy disk.

There are three phases in BACKUP. The first phase reads the directories

of  all  the  hard disks, sorts the file names into  alphabetical order

within  file type, and  prints a master directory by type on  the  list

device.  The program assumes hard disk drives A: and B:. If   there are

more  drives  than  this, change  NODRV to the number of drives, change

DRIVE to add in the extra drive letters, and reassemble.

Next the program attempts to restrict the files it backs up.   Since  a

twenty-six megabyte  disk  could  take  up to forty four double density

floppy disks  to completely  back up,  only essential  files are saved.

Duplicate  files  are  eliminated  saving  only the file on  the lowest

drive  ( A: is saved if the file also exists on B: or C: ).   The  file

is  considered duplicate if the names are the same, the actual contents

are not compared.  Certain file types are  considered  unnecessary  and

are not saved.  PRN, BAK, SYM, HEX, and TMP   are  eliminated  from the

backup list.  To change this list of skipped file types, simply  change

the variable NOSKIP and SKIPFILES within the program and reassemble.

Each file type will then be displayed on the console and the program will

ask  if you want to back up all of the files of this type (enter Y), skip

all of the files of this type ( enter N ), or select certain files within

this type ( enter S ).  If you enter S, each file name within the current

type will  be  displayed  on the  console and the program will ask if you

want to backup this particular file ( enter Y ), skip this file (enter N)

or continue to backup all the rest of the files within this type  without

listing each name (enter C).  This feature is used to restart  the backup

if it was terminated  before  it  completed  during a  previous  run.  At

this time you can also  skip  any  files that  will  not  fit entirely on

one disk.  The program will skip this file also if it  encounters it, but

it will prompt for a blank diskette to try the backup before it discovers

that it cannot be backed up with  this program.   When  the list of files

to backup has been reduced, the list will be printed and the program will

ask if it should begin the copy  procedure  or abort.  This allows you to

rerun if the list of files to backup is not correct.

Finally,  the  backup procedure will begin.   The program uses all of RAM

from  the end  of the file list table to the end of the TPA as the buffer

for copying files.  As the files are finished being copied, the tabled is

decreased and the buffer area is  increased thereby speeding up the  copy

process.  The program will display a message asking to mount a floppy  in

the first  drive ( assumed to be C: but changable with the program )  and

enter a three digit disk ID or "END" to terminate prematurely.   This may

happen if you run out of diskettes!!!

The first file that BACKUP writes on the floppy is a null file.    It has

zero  length so it only takes up one directory entry and no space on  the

disk.   This  null file  has the name  equal to the eight character  date

listed on the  reports  and the file type is the three character disk id.

The purpose of this file  is to  identify  the disk against the directory

listing and to automatically mark the date. The heading line for an index

report will then be printed on the  list device showing  the disk  id and

current  date.    As the files are copied, the  index report  is  created

showing the files copied to each floppy.

While the file is being copied, the file name will be displayed on the

console.  When the file has been completely copied, the file name will

be added to the index report.  If the file will not  fit completely on

the floppy, the file name will be deleted from the floppy directory, a

new floppy requested, and the file recopied to the  new floppy.   This

process continues until the entire file name table  has been processed

using as many floppy disks as required.

Restoring or recovering can be done with PIP.   You can PIP the entire

floppy disk to the hard disk, or using the disk id/file name index,

restore only certain files.

When using PIP to copy any REL files back to the hard disk, be sure to

use the OV option in PIP.

The source code is provided in file BACKUP.ASM for your convenience

in changing certain parameters for your installation.  The variables that

may need to be changed are all on the first page of the source and are

the following:

NODRV      This is the number of hard disk drives.  Set to 2 if you have

one hard disk named A: B:

DRIVES     This is the letters for the drives to search.  Set to AB if

you have one hard disk, and the hard disk is addressed before

any floppy disks.

BACKUPDRV  This is the floppy drive letter that the backup will be copied

to.  Set to C if you have hard disk drives A and B.

NOSKIP     This is the number of file types in the file skip list.

SKIPTYPE   This is the list of file types to skip.  It is normally set

to PRNHEXSYMBAKTMP to skip those six file types.

LINESPAGE  This is the number of lines per page and is used to print the

various directories.

RECLINE    This is the number of file names that will appear on one line

in the directories.

The program can  use the COMPUTIME board to get the date, but the code

is in the program to request the date from the console.  Just change the

equate near the beginning of the program to CLOCK EQU TRUE to use the

clock specific code.

6/23/82...RESTART CAPABILITY ADDED

BY ENTERING BACKUP FILENAME.TYP THE PROGRAM WILL SKIP THE CROSSREFERENCE

REPORT AND POSITION THE LIST TO FILENAME.TYP AND REPROMPT FOR FILES TO

BACKUP FROM THAT POINT.

EXAMPLE A>BACKUP LOAD.COM

#########################################################################################################