The Telemark Assembler (TASM) User's Manual

Version 3.1

February, 1998

Thomas N. Anderson

Squak Valley Software

837 Front Street South,

Issaquah, WA 98027

Compuserve: 73770,3612

Internet: andersontn@acm.org

www.halcyon.com/squakvly/

Copyright (C) 1998 by Squak Valley Software. All rights reserved.

------------------------------------------------------------------------

TABLE OF CONTENTS

* INTRODUCTION

* SHAREWARE

* INVOCATION

* ENVIRONMENT VARIABLES

* EXIT CODES

* SOURCE FILE FORMAT

* EXPRESSIONS

* ASSEMBLER DIRECTIVES

* OBJECT FILE FORMATS

* LISTING FILE FORMAT

* PROM PROGRAMMING

* ERROR MESSAGES

* LIMITATIONS

------------------------------------------------------------------------

INTRODUCTION

The Telemark Assembler (TASM) is a table driven cross assembler for the

MS-DOS and LINUX environments. Assembly source code, written in the

appropriate dialect (generally very close to the manufacturers assembly

language), can be assembled with TASM, and the resulting object code

transferred to the target microprocessor system via PROM or other

mechanisms.

The microprocessor families supported by TASM are:

* 6502

* 6800/6801/68HC11

* 6805

* 8048

* 8051

* 8080/8085, Z80

* TMS32010, TMS320C25

* TMS7000

* 8096/80196

The user so inclined may build tables for other microprocessors. The

descriptions of the various existing tables and instructions on building

new tables are not in this document but can be found in the TASMTABS.HTM

file on the TASM distribution disk.

TASM characteristics include:

1. Powerful expression parsing (17 operators).

2. Supports a subset of the 'C' preprocessor commands.

3. Macro capability (through use of DEFINE directive).

4. Multiple statements per line.

5. Four object file formats: Intel hex, MOS Technology hex, Motorola hex,

binary.

6. Absolute code generation only.

7. Source code available (in C).

8. Uniform syntax across versions for different target machines.

9. Features in support of PROM programming (preset memory, contiguous

block).

10. Supports extended instructions for many of the supported

microprocessor families.

11. Tables read at run time - single TASM executable for all table

versions.

12. Symbol table export for inclusion in subsequent assemblies.

13. Symbol table export file for import with some simulator products.

SHAREWARE

TASM is distributed as shareware. TASM is not in the public domain. The

TASM distribution files may be freely copied (excluding the source code

files) and freely used for the purpose of evaluating the suitability of

TASM for a given purpose. Use of TASM beyond a reasonable evaluation period

requires registration. Prolonged use without registration is unethical.

INVOCATION

TASM can be invoked as follows (optional fields shown in brackets, symbolic

fields in italics):

tasm -pn [-options ...] src_file [obj_file [lst_file [exp_file [sym_file]]]]

Where options can be one or more of the following:

-table     Specify version (table = table designation)

-ttable    Table (alternate form of above)

-aamask    Assembly control (optional error checking)

-b         Produce object in binary (.COM) format

-c         Object file written as a contiguous block

-dmacro    Define a macro (or just a macro label)

-e         Show source lines after macro expansion

-ffillbyte Fill entire memory space with fillbyte (hex)

-gobjtype  Object file (0=Intel Hex, 1=MOS Tech, 2=Motorola,

3=binary,4=Intel Hex (Word))

-h         Produce hex table of the assembled code (in list file)

-i         Ignore case for labels

-l[al]     Produce a label table in the listing

-m         Produce object in MOS Technology format

-oobytes   Bytes per object record (for hex obj formats)

-p[lines]  Page the listing file (lines per page. default=60)

-q         Quiet, disable the listing file

-rkb       Set read buffer size in Kbytes (default 2 Kbytes)

-s         Write a symbol table file

-x[xmask]  Enable extended instruction set (if any)

-y         Time the assembly

The filename parameters are defined as follows:

src_file Source file name

obj_file Object code file name

lst_file Listing file name

exp_file Symbol export file (only if the EXPORT directive is used).

sym_file Symbol table file (only if the -s option or the SYM/AVSYM

directives are used).

The source file must be specified. If not, some usage information is

displayed. Default file names for all the other files are generated if they

are not explicitly provided. The filename is formed by taking the source

filename and changing the extension to one of the following:

Extension File type

.OBJ      Object file

.LST      Listing file

.EXP      Symbol export file

.SYM      Symbol table file

TASM has no built-in instruction set tables. Instruction set definition

files are read at run time. TASM determines which table to use based on the

'-table' field shown above. For example, to assemble the code in a file

called source.asm, one would enter

tasm -48 source.asm   for an 8048 assembly

tasm -65 source.asm   for a 6502 assembly

tasm -51 source.asm   for an 8051 assembly.

tasm -85 source.asm   for an 8085 assembly.

tasm -80 source.asm   for a Z80 assembly.

tasm -05 source.asm   for a 6805 assembly.

tasm -68 source.asm   for a 6800/6801/68HC11 assembly.

tasm -70 source.asm   for a TMS7000 assembly.

tasm -3210 source.asm for a TMS32010 assembly.

tasm -3225 source.asm for a TMS320C25 assembly.

tasm -96 source.asm   for a 8096/80196 assembly

Tables are read from a file named by taking the digits specified after the

'-' and appending it to 'TASM' then appending the '.TAB' extension. Thus,

the -48 flag would cause the tables to be read from the file 'TASM48.TAB'.

It is possible to designate tables by non numeric part numbers if the -t

flag is used. For example, if a user built a table called TASMF8.TAB then

TASM could be invoked as follows:

tasm -tf8 source.asm

Each option flag must be preceded by a dash. Options need not precede the

file names. The various options are described in the sections that follow.

a - Assembly Control

TASM can provide additional error checking by specifying the -a option at

the time of execution. If the -a is provided without a digit following,

then all the available error checking is done. If a digit follows, then it

is used as a mask to determine the error checks to be made. The bits of the

mask are defined as follows:

Bit Option  Default  Description

0    -a1     OFF    Check for apparent illegal use of indirection

1    -a2      ON    Check for unused data in the arguments

2    -a4      ON    Check for duplicate labels

3    -a8     OFF    Check for non-unary operators at start of

expression.

Combinations of the above bits can also be used. For example, -a5 would

enable the checking for illegal indirection and duplicate labels.

Illegal indirection applies to micros that use parenthesis around an

argument to indicate indirection. Since it is always legal to put an extra

pair of parenthesis around any expression (as far as the expression parser

is concerned), the user may think that he/she is indicating indirection for

an instruction that has no indirection and TASM would not complain.

Enabling this checking will result in an error message (warning) whenever

an outer pair of parenthesis is used and the instruction set definition

table does not explicitly indicate that to be a valid form of addressing.

Unused data in arguments applies to cases where a single byte of data is

needed from an argument, but the argument contains more than one byte of

data. If a full sixteen bit address is used in a 'Load Immediate' type

instruction that needs only a single byte, for example, an error message

would be generated. Here is an example (6502 code):

0001   1234                    .org $1234

test.asm line 0002: Unused data in MS byte of argument.

0002   1234 A9 34       start  lda  #start

To make the above checks occur whenever you do an assembly, add a line like

this to your AUTOEXEC.BAT file:

SET TASMOPTS=-a

b - Binary Object Format

This option causes the object file to be written in binary - one byte for

each byte of code/data. Note that no address information is included in the

object file in this format. The contiguous block (-c) output mode is forced

when this option is invoked. This flag is equivalent to -g3.

c - Contiguous Block Output

If this option is specified, then all bytes in the range from the lowest

used byte to the highest will be defined in the object file. Normally, with

the default Intel Hex object format enabled, if the Program Counter (PC)

jumps forward because of an .ORG directive, the bytes skipped over will not

have any value assigned them in the object file. With this option enabled,

no output to the object file occurs until the end of the assembly at which

time the whole block is written. This is useful when using TASM to generate

code that will be put into a PROM so that all locations will have a known

value. This option is often used in conjunction with the -f option to

ensure all unused bytes will have a known value.

d - Define a Macro

Macros are defined on the command line generally to control the assembly of

various IFDEF's that are in the source file. This is a convenient way to

generate various versions of object code from a single source file.

e - Expand Source.

Normally TASM shows lines in the listing file just as they are in the

source file. If macros are in use (via the DEFINE directive) it is

sometimes desirable to see the source lines after expansion. Use the '-e'

flag to accomplish this.

f - Fill Memory.

This option causes the memory image that TASM maintains to be initialized

to the value specified by the two hex characters immediately following the

'f'. TASM maintains a memory image that is a full 64K bytes in size (even

if the target processor cannot utilize that memory space). Invocation of

this option introduces a delay at start up of up to 2 seconds (time

required to initialize all 64K bytes).

g - Object File Format.

TASM can generate object code in four different formats as indicated below:

Option Description

-g0   Intel hex (default)

-g1   MOS Technology hex (same as -m)

-g2   Motorola hex

-g3   binary (same as -b)

-g4   Intel hex with word addresses

The -m and -b flags may also be used, as indicated above. If both are used

the right-most option on the command line will be obeyed.

See the section on OBJECT FILE FORMATS for descriptions of each of the

above.

h - Hex Object Code Table.

This option causes a hex table of the produced object code to appear in the

listing file. Each line of the table shows sixteen bytes of code.

i - Ignore Case in Labels.

TASM is normally case sensitive when dealing with labels. For those that

prefer case insensitivity, the '-i' command line option can be employed.

l - Label Table.

This option causes a label table to appear in the listing file. Each label

is shown with its corresponding value. Macro labels (as established via the

DEFINE directives) do not appear.

Two optional suffixes may follow the -l option:

Suffix Description

l    Use long form listing

a    Show all labels (including local labels)

The suffix should immediately follow the '-l'. Here are some examples:

-l   to show non-local labels in the short form

-la  to show all labels in the short form

-ll  to show non-local labels in the long form

-lal to show all labels in the long form

m - MOS Technology Object Format.

This option causes the object file to be written in MOS Technology hex

format rather than the default Intel hex format. See section on OBJECT FILE

FORMATS for a description of the format.

o - Set Number of Bytes per Object Record.

When generating object code in either the MOS Technology format or the

Intel hex format, a default of 24 (decimal) bytes of object are defined on

each record. This can be altered by invoking the '-o' option immediately

followed by two hex digits defining the number of bytes per record desired.

For example, if 32 bytes per record are desired, one might invoke TASM as:

tasm -48 -o20 source.asm

p - Page Listing File.

This option causes the listing file to have top of page headers and form

feeds inserted at appropriate intervals (every sixty lines of output). To

override the default of sixty lines per page, indicate the desired number

of lines per page as a decimal number immediately following the '-p'. Here

is an example:

tasm -48 -p56 source.asm

q - Disable Listing File.

This option causes all output to the listing file to be suppressed, unless

a .LIST directive is encountered in the source file (see LIST/NOLIST

directives).

r - Set Read Buffer Size.

This option overrides the default read buffer size of 2 Kbytes. The first

hexadecimal digit immediately after the 'r' is taken as the number of K

bytes to allocate for the read buffer (.e.g. -r8 indicates an 8K byte

buffer, -rf indicates a 15K byte buffer). Note that that read buffers are

taken from the same memory pool as labels and macro storage, and that

additional read buffers are needed if "includes" are used. Thus, using 8K

byte buffers may be suitable for most assemblies, but programs with large

numbers of symbols may not allow such a value. Also, reducing the buffer

size to 1 Kbyte can increase the memory pool available for label storage,

if such is needed.

s - Enable Symbol File Generation.

If this flag is set, a symbol file is generated at the end of the assembly.

The format of the file is one line per label, each label starts in the

first column and is followed by white space and then four hexadecimal

digits representing the value of the label. The following illustrates the

format:

label1         FFFE

label2         FFFF

label3         1000

The symbol file name can be provided as the fifth file name on the command

line, or the name will be generated from the source file name with a '.SYM'

extension. The symbol table file can also be generated by invoking the SYM

directive. The AVSYM directive also generates the symbol file but in a

different format (see section on ASSEMBLER DIRECTIVES).

t - Table Name.

As an alternative to specifying the instruction set table as two decimal

digits, the table indication may be proceeded by the '-t' option. This is

useful if the desired table name starts with a non-numeric. Thus, a table

for an F8 might be selected as:

tasm -tf8 source.asm

TASM would expect to read the instruction set definition tables from a file

named TASMF8.TAB.

x - Enable Extended Instruction Set.

If a processor family has instructions that are valid for only certain

members, this option can be used to enable those beyond the basic standard

instruction set. A hex digit may follow the 'x' to indicate a mask value

used in selecting the appropriate instruction set. Bit 0 of the mask

selects the basic instruction set, thus a '-x1' would have no effect. A

'-x3' would enable the basic set plus whatever instructions have bit 1 set

in their class mask. A '-x' without a digit following is equivalent to a

'-xf' which sets all four of the mask bits. The following table indicates

the current extended instruction sets available in the TASM tables:

Base Table Base Family  Ext 1 (-x3)   Ext 2 (-x7)Ext 3 (-x5)  Ext 4 (-x9)

48         8048         8041A                    8022         8021

65         6502         R65C02                   R65C00/21

05         6805         M146805 CMOS             HC05C4

80         Z80          HD64180

68         6800         6801/6803     68HC11

51         8051

85         8080

3210       TMS32010

3225       TMS320C25                             TMS320C26

70         TMS7000

The above table does not attempt to show the many microprocessor family

members that may apply under a given column.

See the TASMTABS.TXT on-line document for details on each specific table.

y - Enable Assembly Timing

If this option is enabled TASM will generate a statement of elapsed time

and assembled lines per second at the end of the assembly.

ENVIRONMENT VARIABLES

The TASM environment can be customized by using the environment variables

listed below:

TASMTABS

The TASMTABS variable specifies the path to be searched for TASM

instruction set definition tables. If it is not defined then the table(s)

must exist in the current working directory. The following examples

illustrate possible usage:

For MSDOS set TASMTABS=C:\TASM

For LINUX TASMTABS=/tasm

TASMOPTS

This variable specifies TASM command line options that are to be invoked

every time TASM is executed. For example, if TASM is being used for 8048

assemblies with binary object file output desired, the following statement

would be appropriate in the AUTOEXEC.BAT file:

set TASMOPTS=-48 -b

EXIT CODES

When TASM terminates, it will return to the OS the following exit codes:

Exit Code Definition

0     Normal completion, no assembly errors

1     Normal completion, with assembly errors

2     Abnormal completion, insufficient memory

3     Abnormal completion, file access error

4     Abnormal completion, general error

Exit codes 2 and above will also be accompanied by messages to the console

concerning the error.

SOURCE FILE FORMAT

Statements in the source file must conform to a format as follows (except

for assembler directive statements which are described in a subsequent

section):

label operation operand comment

All of the fields are optional, under appropriate circumstances. An

arbitrary amount of white space (space and tabs) can separate each field

(as long as the maximum line length of 255 characters is not exceeded).

Each of the fields are described in the following sections.

Label Field.

If the first character of the line is alphabetic, it is assumed to be the

start of a label. Subsequent characters are accepted as part of that label

until a space, tab, or ':' is encountered. The assembler assigns a value to

the label corresponding to the current location counter. Labels can be a

maximum of 32 characters long. Labels can contain upper and lower case

letters, digits, underscores, and periods (the first character must be

alphabetic). Labels are case sensitive - the label 'START' is a different

label from 'start' - unless the '-i' (ignore case) option is enabled.

Operation Field.

The operation field contains an instruction mnemonic which specifies the

action to be carried out by the target processor when this instruction is

executed. The interpretation of each mnemonic is dependent on the target

microprocessor (as indicated by the selected TASM table). The operation

field may begin in any column except the first. The operation field is case

insensitive.

Operand Field.

The operand field specifies the data to be operated on by the instruction.

It may include expressions and/or special symbols describing the addressing

mode to be used. The actual format and interpretation is dependent on the

target processor. For a description of the format for currently supported

processors, see the TASMTABS.DOC file on the TASM distribution disk.

Comment Field.

The comment field always begins with a semicolon. The rest of the line from

the semicolon to the end of the line is ignored by TASM, but passed on to

the listing file for annotation purposes. The comment field must be the

last field on a line, but it may be the only field, starting in column one,

if desired.

Multiple Statement Lines.

If the backslash character is encountered on a source line, it is treated

as a newline. The remainder of the line following the backslash will be

processed as an independent line of source code. This allows one to put

multiple statements on a line. This facility is not so useful of itself,

but when coupled with the capability of the DEFINE directive, powerful

multiple statement macros can be constructed (see section on ASSEMBLER

DIRECTIVES). Note that when using the statement separator, the character

immediately following it should be considered the first character of a new

line, and thus must either be a start of a label or white space (not an

instruction). As the examples show, a space is put between the backslash

and the start of the next instruction.

Sample Source Listing.

Some examples of valid source statements follow (6502 mnemonics shown):

lab1       lda    byte1   ;get the first byte

dec    byte1

jne    label1

;

lab2       sta    byte2,X

;  a multiple statement line follows

lda    byte1\ sta byte1+4\ lda byte2\ sta byte2+4

EXPRESSIONS

Expressions are made up of various syntactic elements combined according to

a set of syntactical rules. Expressions can be comprised of the following

elements:

* Labels

* Constants

* Location Counter Symbol

* Operators

* Parenthesis

Labels

Labels are strings of characters that have a numeric value associated with

them, generally representing an address. Labels can contain upper and lower

case letters, digits, underscores, and periods. The first character must be

a letter or the local label prefix (default '_'). The value of a label is

limited to 32 bit precision. Labels can contain up to 32 characters, all of

which are significant (none are ignored when looking at a label's value, as

in some assemblers). Case is significant unless the '-i' command line

option is invoked.

Local labels must only be unique within the scope of the current module.

Modules are defined with the MODULE directive. Here is an example:

.MODULE xxx

lda regx

jne _skip

dec

_skip  rts

.MODULE yyy

lda regy

jne _skip

dec

_skip  rts

In the above example, the _skip label is reused without harm. As a default,

local labels are not shown in the label table listing (resulting from the

'-l' command line option). See also sections on MODULE and LOCALLABELCHAR

directives.

Numeric Constants

Numeric constants must always begin with a decimal digit (thus hexadecimal

constants that start with a letter must be prefixed by a '0' unless the '$'

prefix is used). The radix is determined by a letter immediately following

the digit string according to the following table:

Radix Suffix               Prefix

2   B or b                  %

8   O or o                  @

10   D or d (or nothing)

16   H or h                  $

Decimal is the default radix, so decimal constants need no suffix or

prefix.

The following representations are equivalent:

1234H          or     $1234

100d           or     100

177400o        or     @177400

01011000b      or     %01011000

The prefixes are provided for compatibility with some other source code

formats but introduce a problem of ambiguity. Both '%' and '$' have

alternate uses ('%' for modulo, '$' for location counter symbol). The

ambiguity is resolved by examining the context. The '%' character is

interpreted as the modulo operator only if it is in a position suitable for

a binary operator. Similarly, if the first character following a '$' is a

valid hexadecimal digit, it is assumed to be a radix specifier and not the

location counter.

Character Constants

Character constants are single characters surrounded by single quotes. The

ASCII value of the character in the quotes is returned. No escape provision

exists to represent non-printable characters within the quotes, but this is

not necessary since these can be just as easily represented as numeric

constants (or using the TEXT directive which does allow escapes).

String Constants.

String constants are one or more characters surrounded by double quotes.

Note that string constants are not allowed in expressions. They are only

allowable following the TITLE, BYTE, DB, and TEXT assembler directives. The

quoted strings may also contain escape sequences to put in unprintable

values. The following escape sequences are supported:

Escape Sequence Description

\n        Line Feed

\r        Carriage return

\b        Backspace

\t        Tab

\f        Formfeed

\\        Backslash

\"        Quote

\000       Octal value of character

Location Counter Symbol

The current value of the location counter (PC) can be used in expressions

by placing a '$' in the desired place. The Location Counter Symbol is

allowable anywhere a numeric constant is. (Note that if the '$' is followed

by a decimal digit then it is taken to be the hexadecimal radix indicator

instead of the Location Counter symbol, as mentioned above). The '*' may

also be used to represent the location counter, but is less preferred

because of its ambiguity with the multiplicative operator.

Operators

Expressions can optionally contain operators to perform some alterations or

calculations on particular values. The operators are summarized as follows:

Operator      Type       Description

+        Additive     addition

-                     subtraction

*     Multiplicative  multiplication

/                     division

%                     modulo

<<                    logical shift left

>>                    logical shift right

~     Unary           bit inversion (one's complement)

-                     unary negation

=     Relational      equal

==                    equal

!=                    not equal

<                    less than

>                    greater than

<=                    less than or equal

>=                    greater than or equal

&    Binary          binary 'and'

|                     binary 'or'

^                     binary 'exclusive or'

The syntax is much the same as in 'C' with the following notes:

1. No operator precedence is in effect. Evaluation is from left to right

unless grouped by parenthesis (see example below).

2. All evaluations are done with 32 bit signed precision.

3. Both '=' and '==' are allowable equality checkers. This is allowed

since the syntax does not provide assignment capability (as '=' would

normally imply).

The relational operators return a value of 1 if the relation is true and 0

if it is false. Thirty-two bit signed arithmetic is used.

It is always a good idea to explicitly indicate the desired order of

evaluation with parenthesis, especially to maintain portability since TASM

does not evaluate expressions in the same manner as many other assemblers.

To understand how it does arrive at the values for expressions, consider

the following example:

1 + 2*3 + 4

TASM would evaluate this as:

(((1 + 2) * 3) + 4) = 13

Typical rules of precedence would cause the (2*3) to be evaluated first,

such as:

1 + (2*3) + 4      = 11

To make sure you get the desired order of evaluation, use parenthesis

liberally. Here are some examples of valid expressions:

(0f800H + tab)

(label_2 >> 8)

(label_3 << 8) & $f000

$ + 4

010010000100100b + 'a'

(base + ((label_4 >> 5) & (mask << 2))

ASSEMBLER DIRECTIVES

Most of the assembler directives have a format similar to the machine

instruction format. However, instead of specifying operations for the

processor to carry out, the directives cause the assembler to perform some

function related to the assembly process. TASM has two types of assembler

directives - those that mimic the 'C' preprocessor functions, and those

that resemble the more traditional assembler directive functions. Each of

these will be discussed.

The 'C' preprocessor style directives are invoked with a '#' as the first

character of the line followed by the appropriate directive (just as in

'C'). Thus, these directives cannot have a label preceding them (on the

same line). Note that in the examples directives are shown in upper case,

however, either upper or lower case is acceptable.

ADDINSTR

The ADDINSTR directive can be used to define additional instructions for

TASM to use in this assembly. The format is:

[label] .ADDINSTR inst args opcode nbytes rule class shift binor

The fields are separated by white space just as they would appear in an

instruction definition file. See the TASMTABS.TXT file on the TASM

distribution disk for more detail.

AVSYM

See SYM/AVSYM.

BLOCK

The BLOCK directive causes the Instruction Pointer to advance the specified

number of bytes without assigning values to the skipped over locations. The

format is:

[label] .BLOCK        expr

Some valid examples are:

word1   .BLOCK     2

byte1   .block     1

buffer  .block     80

BSEG/CSEG/DSEG/NSEG/XSEG

These directives can be invoked to indicate the appropriate address space

for symbols and labels defined in the subsequent code. The invocation of

these directives in no way affects the code generated, only provides more

information in the symbol table file if the AVSYM directive is employed.

Segment control directives such as these are generally supported by

assemblers that generate relocatable object code. TASM does not generate

relocatable object code and does not support a link phase, so these

directives have no direct effect on the resulting object code. The segments

are defined as follows:

Directive Segment Description

BSEG    Bit address

CSEG    Code address

DSEG    Data address (internal RAM)

NSEG    Number or constant (EQU)

XSEG    External data address (external RAM)

BYTE

The BYTE directive allows a value assignment to the byte pointed to by the

current Instruction Pointer. The format is:

[label] .BYTE   expr [, expr ...]

Only the lower eight bits of expr are used. Multiple bytes may be assigned

by separating them with commas or (for printable strings) enclosed in

double quotes. Here are some examples:

label1   .BYTE     10010110B

.byte     'a'

.byte     0

.byte     100010110b,'a',0

.byte     "Hello", 10, 13, "World"

CHK

The CHK directive causes a checksum to be computed and deposited at the

current location. The starting point of the checksum calculation is

indicated as an argument. Here is the format:

[label]    .CHK    starting_addr

Here is an example:

start: NOP

LDA #1

.CHK start

The checksum is calculated as the simple arithmetic sum of all bytes

starting at the starting_addr up to but not including the address of the

CHK directive. The least significant byte is all that is used.

CODES/NOCODES

The CODES/NOCODES directives can be used to alternately turn on or off the

generation of formatted listing output with line numbers, opcodes, data,

etc. With NOCODES in effect, the source lines are sent to the listing file

untouched. This is useful around blocks of comments that need a full 80

columns of width for clarity.

DB

This is alternate form of the BYTE directive.

DW

This is alternate form of the WORD directive.

DEFINE

The DEFINE directive is one of the most powerful of the directives and

allows string substitution with optional arguments (macros). The format is

as follows:

#DEFINE  macro_label[(arg_list)]  [macro_definition]

Where:

macro_label

character string to be expanded when found in the source file

arg_list

optional argument list for variable substitution in macro expansion

macro_def

string to replace the occurrences of macro_label in the source file.

The simplest form of the DEFINE directive might look like this:

#DEFINE         MLABEL

Notice that no substitutionary string is specified. The purpose of a

statement like this would typically be to define a label for the purpose of

controlling some subsequent conditional assembly (IFDEF or IFNDEF).

A more complicated example, performing simple substitution, might look like

this:

#DEFINE         VAR1_LO         (VAR1 & 255)

This statement would cause all occurrences of the string 'VAR1_LO' in the

source to be substituted with '(VAR1 & 255)'.

As a more complicated example, using the argument expansion capability,

consider this:

#DEFINE  ADD(xx,yy)    clc\ lda xx\ adc yy\ sta xx

If the source file then contained a line like this:

ADD(VARX,VARY)

It would be expanded to:

clc\ lda VARX\ adc VARY\ sta VARX

The above example shows the use of the backslash ('\') character as a

multiple instruction statement delimiter. This approach allows the

definition of fairly powerful, multiple statement macros. The example shown

generates 6502 instructions to add one memory location to another.

Some rules associated with the argument list:

1. Use a maximum of 10 arguments.

2. Each argument should be a maximum of 15 characters.

Note that macros can be defined on the TASM command line, also, with the -d

option flag.

DEFCONT

The DEFCONT directive can be used to add to the last macro started with a

DEFINE directive. This provides a convenient way to define long macros

without running off the edge of the page. The ADD macro shown above could

be defined as follows:

#DEFINE         ADD(xx,yy)     clc

#DEFCONT                     \ lda xx

#DEFCONT                     \ adc yy

#DEFCONT                     \ sta xx

ECHO

The ECHO directive can be used to send output to the console (stderr). It

can accept either a quoted text string (with the standard escape sequences

allowed) or a valid expression. It can accept only one or the other,

however. Multiple instances of the directive may be used to create output

that contains both. Consider the following example:

.ECHO "The size of the table is "

.ECHO (table_end - table_start)

.ECHO " bytes long.\n"

This would result in a single line of output something like this:

The size of the table is 196 bytes long.

EJECT

The EJECT directive can be used to force a top-of-form and the generation

of a page header on the list file. It has no effect if the paging mode is

off (see PAGE/NOPAGE). The format is:

[label]    .EJECT

ELSE

The ELSE directive can optionally be used with IFDEF, IFNDEF and IF to

delineate an alternate block of code to be assembled if the block

immediately following the IFDEF, IFNDEF or IF is not assembled.

Here are some examples of the use of IFDEF, IFNDEF, IF, ELSE, and ENDIF:

#IFDEF   label1

lda      byte1

sta      byte2

#ENDIF

#ifdef   label1

lda      byte1

#else

lda      byte2

#endif

#ifndef  label1

lda      byte2

#else

lda      byte1

#endif

#if ($ >= 1000h)

; generate an invalid statement to cause an error

;  when we go over the 4K boundary.

!!! PROM bounds exceeded.

#endif

END

The END directive should follow all code/data generating statements in the

source file. It forces the last record to be written to the object file.

The format is:

[label]       .END [addr]

The optional addr will appear in the last object record (Motorola S9 record

type) if the object format is Motorola hex. The addr field is ignored for

all other object formats.

ENDIF

The ENDIF directive must always follow an IFDEF, IFNDEF, or IF directive

and signifies the end of the conditional block.

EQU

The EQU directive can be used to assign values to labels. The labels can

then be used in expressions in place of the literal constant. The format

is:

label   .EQU   expr

Here is an example:

MASK   .EQU  0F0H

;

lda   IN_BYTE

and   MASK

sta   OUT_BYTE

An alternate form of the EQU directive is '='. The previous example is

equivalent to any of the following:

MASK    =   0F0H

MASK    =0F0H

MASK    =$F0

White space must exist after the label, but none is required after the '='.

EXPORT

The EXPORT directive can be used to define labels (symbols) that are to be

written to the export symbol file. The symbols are written as equates

(using the .EQU directive) so that the resulting file can be included in a

subsequent assembly. This feature can help overcome some of the

deficiencies of TASM due to its lack of a relocating linker. The format is:

[label]  .EXPORT      label [,label...]

The following example illustrates the use of the EXPORT directive and the

format of the resulting export file:

Source file:

EXPORT        read_byte

EXPORT        write_byte, open_file

Resulting export file:

read_byte      .EQU   $1243

write_byte     .EQU   $12AF

open_file      .EQU   $1301

FILL

The FILL directive can be used to fill a selected number of object bytes

with a fixed value. Object memory is filled from the current program

counter forward. The format is as follows:

[label]  .FILL      number_of_bytes [,fill_value]

The number_of_bytes value can be provided as any valid expression. The

optional fill_value can also be any valid expression. If fill_value is not

provided, a default value of 255 ($FF) is used.

IFDEF

The IFDEF directive can be used to optionally assemble a block of code. It

has the following form:

#IFDEF  macro_label

When invoked, the list of macro labels (established via DEFINE directives)

is searched. If the label is found, the following lines of code are

assembled. If not found, the input file is skipped until an ENDIF or ELSE

directive is found.

Lines that are skipped over still appear in the listing file, but a '~'

will appear immediately after the current PC and no object code will be

generated (this is applicable to IFDEF, IFNDEF, and IF).

IFNDEF

The IFNDEF directive is the opposite of the IFDEF directive. The block of

code following is assembled only if the specified macro_label is undefined.

It has the following form:

#IFNDEF  macro_label

When invoked, the list of macro labels (established via DEFINE directives)

is searched. If the label is not found, the following lines of code are

assembled. If it is found, the input file is skipped until an ENDIF or ELSE

directive is found.

IF

The IF directive can be used to optionally assemble a block of code

dependent on the value of a given expression. The format is as follows:

#IF     expr

If the expression expr evaluates to non-zero, the following block of code

is assembled (until an ENDIF or ELSE is encountered).

INCLUDE

The INCLUDE directive reads in and assembles the indicated source file.

INCLUDEs can be nested up to six levels. This allows a convenient means to

keep common definitions, declarations, or subroutines in files to be

included as needed. The format is as follows:

#INCLUDE        filename

The filename must be enclosed in double quotes. Here are some examples:

#INCLUDE       "macros.h"

#include       "equates"

#include       "subs.asm"

LIST/NOLIST

The LIST and NOLIST directives can be used to alternately turn the output

to the list file on (LIST) or off (NOLIST). The formats are:

.LIST

.NOLIST

LOCALLABELCHAR

The LOCALLABELCHAR directive can be used to override the default "_" as the

label prefix indicating a local label. For example, to change the prefix to

"?" do this:

[label]   .LOCALLABELCHAR "?"

Be careful to use only characters that are not operators for expression

evaluation. To do so causes ambiguity for the expression evaluator. Some

safe characters are "?", "{", and "}".

LSFIRST/MSFIRST

The LSFIRST and MSFIRST directives determine the byte order rule to be

employed for the WORD directive. The default (whether correct or not) for

all TASM versions is the least significant byte first (LSFIRST). The

following illustrates its effect:

0000  34 12    .word $1234

0002           .msfirst

0002  12 34    .word $1234

0004           .lsfirst

0004  34 12    .word $1234

MODULE

The MODULE directive defines the scope of local labels. The format is:

[label]   .MODULE label

Here is an example:

.MODULE module_x

lda regx

jne _skip

dec

_skip  rts

.MODULE module_y

lda regy

jne _skip

dec

_skip  rts

In the above example, the local label _skip is reused without harm since

the two usages are in separate modules. See also section LOCALLABELCHAR

directive.

ORG

The ORG directive provides the means to set the Instruction Pointer (a.k.a.

Program Counter) to the desired value. The format is:

[label] .ORG    expr

The label is optional. The Instruction pointer is assigned the value of the

expr. For example, to generate code starting at address 1000H, the

following could be done:

start   .ORG    1000H

The expression (expr) may contain references to the current Instruction

Pointer, thus allowing various manipulations to be done. For example, to

align the Instruction Pointer on the next 256 byte boundary, the following

could be done:

.ORG  (($ + 0FFH) & 0FF00H)

ORG can also be used to reserve space without assigning values:

.ORG    $+8

An alternate form of ORG is '*=' or '$='. Thus the following two examples

are exactly equivalent to the previous example:

*=*+8

$=$+8

PAGE/NOPAGE

The PAGE/NOPAGE directives can be used to alternately turn the paging mode

on (PAGE) or off (NOPAGE). If paging is in effect, then every sixty lines

of output will be followed by a Top of Form character and a two line header

containing page number, filename, and the title. The format is:

.PAGE

.NOPAGE

The number of lines per page can be set with the '-p' command line option.

SET

The SET directive allows the value of an existing label to be changed. The

format is:

label   .SET    expr

The use of the SET directive should be avoided since changing the value of

a label can sometimes cause phase errors between pass 1 and pass 2 of the

assembly.

SYM/AVSYM

These directives can be used to cause a symbol table file to be generated.

The format is:

.SYM    ["symbol_filename"]

.AVSYM  ["symbol_filename"]

For example:

.SYM       "symbol.map"

.SYM

.AVSYM     "prog.sym"

.AVSYM

The two directives are similar, but result in a different format of the

symbol table file. The format of the SYM file is one line per symbol, each

symbol starts in the first column and is followed by white space and then

four hexadecimal digits representing the value of the symbol. The following

illustrates the format:

label1         FFFE

label2         FFFF

label3         1000

The AVSYM directive is provided to generate symbol tables compatible with

the Avocet 8051 simulator. The format is similar, but each line is prefixed

by an 'AS' and each symbol value is prefixed by a segment indicator:

AS     start          C:1000

AS     read_byte      C:1245

AS     write_byte     C:1280

AS     low_nib_mask   N:000F

AS     buffer         X:0080

The segment prefixes are determined by the most recent segment directive

invoked (see BSEG/CSEG/DSEG/NSEG/XSEG directives).

TEXT

This directive allows an ASCII string to be used to assign values to a

sequence of locations starting at the current Instruction Pointer. The

format is:

[label] .TEXT   "string"

The ASCII value of each character in string is taken and assigned to the

next sequential location. Some escape sequences are supported as follows:

Escape Sequence Description

\n        Line Feed

\r        Carriage return

\b        Backspace

\t        Tab

\f        Formfeed

\\        Backslash

\"        Quote

\000       Octal value of character

Here are some examples:

message1   .TEXT   "Disk I/O error"

message2   .text   "Enter file name "

.text   "abcdefg\n\r"

.text   "I said \"NO\""

TITLE

The TITLE directive allows the user to define a title string that appears

at the top of each page of the list file (assuming the PAGE mode is on).

The format is:

[label]    .TITLE  "string"

The string should not exceed 80 characters. Here are some examples:

.TITLE  "Controller version 1.1"

.title  "This is the title of the assembly"

.title  ""

WORD

The WORD directive allows a value assignment to the next two bytes pointed

to by the current Instruction Pointer. The format is:

[label] .WORD expr [,expr...]

The least significant byte of expr is put at the current Instruction

Pointer with the most significant byte at the next sequential location

(unless the MSFIRST directive has been invoked). Here are some examples:

data_table     .WORD   (data_table + 1)

.word   $1234

.Word   (('x' - 'a')  << 2)

.Word  12, 55, 32

------------------------------------------------------------------------

OBJECT FILE FORMATS

Intel Hex Object Format

This is the default object file format. This format is line oriented and

uses only printable ASCII characters except for the carriage return/line

feed at the end of each line. The format is symbolically represented as:

:NN AAAA RR HH CC CRLF

Where:

:   Record Start Character (colon)

NN  Byte Count (2 hex digits)

AAAAAddress of first byte (4 hex digits)

RR  Record Type (00 except for last record which is 01)

HH  Data Bytes (a pair of hex digits for each byte of data in the record)

CC  Check Sum (2 hex digits)

CRLFLine Terminator (CR/LF for DOS, LF for LINUX)

The last line of the file will be a record conforming to the above format

with a byte count of zero.

The checksum is defined as:

sum = byte_count+address_hi+address_lo+record_type+(sum of all data bytes)

checksum = ((-sum) & ffh)

Here is a sample listing file followed by the resulting object file:

0001   0000

0002   1000                     .org   $1000

0003   1000 010203040506        .byte  1, 2, 3, 4, 5, 6, 7, 8

0003   1006 0708

0004   1008 090A0B0C0D0E        .byte  9,10,11,12,13,14,15,16

0004   100E 0F10

0005   1010 111213141516        .byte 17,18,19,20,21,22,23,24,25,26

0005   1016 1718191A

0006   101A                     .end

:181000000102030405060708090A0B0C0D0E0F101112131415161718AC

:02101800191AA3

:00000001FF

Intel Hex Word Address Object Format

This format is identical to the Intel Hex Object Format except that the

address for each line of object code is divided by two thus converting it

to a word address (16 bit word). All other fields are identical.

Here is an example:

:180800000102030405060708090A0B0C0D0E0F101112131415161718AC

:02080C00191AA3

:00000001FF

MOS Technology Hex Object Format

This format is line oriented and uses only printable ASCII characters

except for the carriage return/line feed at the end of each line. Each line

in the file is of the following format:

:NN AAAA HH CC CRLF

Where:

;   Record Start Character (semicolon)

NN  Byte Count (2 hex digits)

AAAAAddress of first byte (4 hex digits)

HH  Data Bytes (a pair of hex digits for each byte of data in the record)

CCCCCheck Sum (4 hex digits)

CRLFLine Terminator (CR/LF for DOS, LF for LINUX)

The last line of the file will be a record conforming to the above format

with a byte count of zero.

The checksum is defined as:

sum =byte_count+address_hi+address_lo+record_type+(sum of all data bytes)

checksum = (sum & ffffh)

Here is a sample object file:

;1810000102030405060708090A0B0C0D0E0F1011121314151617180154

;021018191A005D

;00

Motorola Hex Object Format

This format is line oriented and uses only printable ASCII characters

except for the carriage return/line feed at the end of each line. The

format is symbolically represented as:

S1 NN AAAA HH CCCC CRLF

Where:

S1  Record Start tag

NN  Byte Count (2 hex digits) (data byte count + 3)

AAAAAddress of first byte (4 hex digits)

HH  Data Bytes (a pair of hex digits for each byte of data in the record)

CC  Check Sum (2 hex digits)

CRLFLine Terminator (CR/LF for DOS, LF for LINUX)

The checksum is defined as:

sum = byte_count+address_hi+address_lo+(sum of all data bytes)

checksum = ((~sum) & ffh)

Here is a sample file:

S11B10000102030405060708090A0B0C0D0E0F101112131415161718A8

S1051018191A9F

S9030000FC

The last line of the file will be a record with a byte count of zero and a

tag of S9. The address field will be 0000 unless and address was provided

with the END directive in which case it will appear in the address field.

Binary Object Format.

This file format is essentially a memory image of the object code without

address, checksum or format description information.

Note that when this object format is selected (-b option), the -c option is

forced. This is done so that no ambiguity results from the lack of address

information in the file. Without the -c option, discontinuous blocks of

object code would appear contiguous.

------------------------------------------------------------------------

LISTING FILE FORMAT

Each line of source code generates one (or more) lines of output in the

listing file. The fields of the output line are as follows:

1. Current source file line number (4 decimal digits).

2. An optional '+' appears if this is an 'INCLUDE' file. (One '+' for

each level of INCLUDE invoked).

3. Current Instruction Pointer (4 hex digits). An optional '~' follows

the Instruction Pointer if the line of source code is not being

assembled because of an IFDEF, IFNDEF, or IF directive.

4. Resulting code/data generated from this source line (two hex digits

per byte, each byte separated by a space, up to six bytes per line).

5. The source line exactly as it appears in the source file.

If paging is enabled (by either the '-p' option flag or the .PAGE

directive) some additional fields will be inserted into the listing file

every 60 lines. These fields are:

1. Top of Form (form feed).

2. Assembler identifier (e.g. "TASM 6502 Assembler").

3. Initial source file name.

4. Page number.

5. Title.

If errors are encountered, then error messages will be interspersed in the

listing. TASM outputs error messages proceeding the offending line. The

following example illustrates the format:

0001   0000             label1  .equ  40h

0002   0000             label2  .equ  44h

0003   0000

0004   1000             start:  .org  1000h

0005   1000 E6 40               inc   label1

0006   1002 E6 44               inc   label2

tt.asm line 0007: Label not found: (label3)

0007   1004 EE 00 00            inc   label3

0008   1007 4C 00 10            jmp   start

0009   100A                     .end

0010   100A

tasm: Number of errors = 1

------------------------------------------------------------------------

PROM PROGRAMMING

A wide variety of PROM programming equipment is available that can use

object code in one or more of the formats supported by TASM. Here are some

notes concerning the generation of code to be programmed into PROMs:

PRESET MEMORY

It is often desirable to have all bytes in the PROM programmed even if not

explicitly assigned a value in the source code (e.g. the bytes are skipped

over with a .ORG statement). This can be accomplished by using the -c

(contiguous block) and the -f (fill) command line option flags. The -c will

ensure that every byte from the lowest byte assigned a value to the highest

byte assigned a value will be in the object file with no gaps. The -f flag

will assign the specified value to all bytes before the assembly begins so

that when the object file is written, all bytes not assigned a value in the

source code will have a known value. As an example, the following command

line will generate object code in the default Intel Hex format with all

bytes not assigned a value in the source set to EA (hex, 6502 NOP

instruction):

tasm -65 -c -fEA test.asm

CONTIGUOUS BLOCKS

To ensure that TASM generates object code to cover the full address range

of the target PROM, put a .ORG statement at the end of the source file set

to the last address desired. For example, to generate code to be put in a

2716 EPROM (2 Kbytes) from hex address $1000 to $17ff, do something like

this in the source file:

;start of the file

.ORG    $1000

;rest of the source code follows

source code

;end of the source code

.ORG    $17ff

.BYTE   0

.END

Now, to invoke TASM to generate the code in the binary format with all

unassigned bytes set to 00 (6502 BRK instruction), do the following:

tasm -65 -b -f00 test.asm

Note that -b forces the -c option.

------------------------------------------------------------------------

ERROR MESSAGES

Error Message Format

TASM error messages take the following general form:

filename line line_number: error_message

For example:

main.asm line 0032: Duplicate label (start)

This format is compatible with the Brief editor (from Borland

International). Brief provides the ability to run assemblies from within

the editor. Upon completion of the assembly, Brief will parse the error

messages and jump to each offending line in the source file allowing the

user to make corrective edits.

To use this feature, it is necessary to configure a Brief environment

variable to specify the assembly command associated with source files that

end in .asm:

SET BCASM = "tasm %s.asm"

TASM also needs to know the proper table to use. It can be added above, or

the TASMOPTS environment variable can be used separately:

SET TASMOPTS=-65

ERROR MESSAGE DESCRIPTIONS

Binary operator where value expected

Two binary operators in a row indicate a missing value.

Branch off of current 2K page

An instruction is attempting to branch to a location not within the

current 2K byte page.

Branch off of current page

An instruction is attempting to branch to a location not within the

current 256 byte page.

Cannot malloc for label storage

Insufficient memory to store more labels. See LIMITATIONS.

Duplicate label

The label for the current line has already been assigned a value.

Duplicate label checks are optionally enabled by the -a option.

File name too short

A file name on the command line is fewer than 3 characters. A two

character file name may be valid, of course, but it is detected as an

error to prevent a garbled option flag from being taken as a source

file, which in turn can result in the true source file being taken as

the object file. Since the object file is truncated at startup time,

the source file could be clobbered.

Forward reference in equate

An EQU directive is using a label on the right hand side that has not

yet been defined.

Heap overflow on label definition

TASM was unable to allocate memory to store the label.

Imbalanced conditional.

An end-of-file was encountered at which time the level of descent in

conditional directives was different from when the file was entered.

Conditional directives include IF, IFDEF, and IFNDEF.

Invalid Object file type.

An object file type was requested by the -g command line option that

is not valid. See section on Option g - Object File Format.

Invalid operand.

No indirection for this instruction. The first character of an operand

was a left parenthesis for an instruction that does not explicitly

specify that as the format. Some micros use the parenthesis as an

indicator of indirection, but putting a layer of parenthesis around an

expression is always a valid thing to do (as far as the expression

evaluator is concerned). The test for this case is only done if the

-a4 option is selected. See section on ASSEMBLY CONTROL.

Invalid token where value expected.

Two binary operators in a row are not allowed.

Label too long.

Labels are limited to 31 characters.

Label value misaligned

The value of a label appears to have a different value on the second

pass then it was computed to have on the first pass. This is generally

due to Zero Page Addressing mode problems with the 6502 version of

TASM. Labels that are used in operands for statements that could

utilize Zero Page addressing mode should always be defined before used

as an operand.

Label not found

A label used in an expression was not found in the current label

table.

Label must pre-exist for SET.

The SET directive can only be applied to an existing label.

Label table overflow

To many labels have been encountered.

List file open error

TASM was not able to open the specified list file.

Macro expansion too long.

The expansion of a macro resulted in a line that exceeded the maximum

length.

Max number of nested conditionals exceeded

Too many levels of IF, IFDEF, or IFNDEF.

Maximum number of args exceeded

Too many macro arguments.

Maximum number of macros exceeded

Too many macros (DEFINEs) have been encountered.

No END directive before EOF

The source file did not have an END directive in it. This is not

fatal, but may cause the last object file record to be lost.

No files specified

TASM was invoked with no source file specified.

No such label

A SET directive was encountered for a label not yet defined. The value

of labels that are modified by the SET directive must already exist.

No terminating quote

A double quote was used at the start of a a text string but was not

used at the end of the string.

No indirection for this instruction.

A parenthesis was found around the operand expression. This may

indicate an attempt to use indirection where it is inappropriate.

Non-unary operator at start of expression

A binary operator (such as '*') was found at the beginning of an

expression. Some micros use '*' as an indirection operator. Since it

is also a legitimate operator in an expression, some ambiguity can

arise. If a particular instruction/addressing mode does not allow

indirection, and a '*' is placed in front of the associated

expression, the assembler will assume this error. See the -a8 option

of ASSEMBLY CONTROL.

Object file open error

TASM was not able to open the specified object file.

Range of argument exceeded

The value of an argument exceeds the valid range for the current

instruction and addressing mode.

Range of relative branch exceeded

A branch instruction exceeds the maximum range.

Source file open error

TASM was not able to open the specified source file.

Unrecognized directive

A statement starting with a '.' or '#' has a mnemonic that is not

defined as a directive.

Unrecognized instruction

A statement has an opcode mnemonic that is not defined.

Unrecognized argument

A statement has an operand format that is not defined.

Unknown token

Unexpected characters were encountered while parsing an expression.

Unused data in MS byte of argument

An instruction or directive used the least significant byte of an

argument and left the most significant byte unused, but it was

non-zero.

Unknown option Flag.

Invalid option flag has been specified on the command line. invoke

TASM with nothing on the command line to see a list of valid options.

LIMITATIONS

Maximum number of labels                   15000

Maximum length of labels                   32 characters

Maximum address space                      64 Kbytes (65536 bytes)

Maximum number of nested INCLUDES          4

Maximum length of TITLE string             79 characters

Maximum source line length                 255 characters

Maximum length after macro expansion       255 characters

Maximum length of expressions              255 characters

Maximum length of pathnames                79 characters

Maximum length of command line             127 characters

Maximum number of instructions (per table) 1200

Maximum number of macros                   1000

Maximum number of macro arguments          10

Maximum length of macro argument           16 characters

Memory requirements                        512K

Other Limitations

1. The 8048 version of TASM does not check for use of memory beyond any

reasonable bounds (e.g. an 8048 has a maximum address space of 4

Kbytes but TASM will let you pretend that you have 64 Kbytes).

2. Expression evaluation has no operator precedence in effect which can

make for unexpected results if not explicitly grouped with

parenthesis.

3. First page of listing file will not show a user defined title (defined

via TITLE directive).

------------------------------------------------------------------------