Applied Microsystems Corporation

XICE68K Version 6.00

for EL 3200 Emulators

on SPARC Workstations

Release Notes

August 26, 1992

===================

COMPATIBILITY NOTES

===================

XICE68K 6.00 requires Sun4 running SunOS 4.1.1 or above.

The configuration file, XICE.CFG, for this release is not compatible with

versions of this file from previous releases.  Do not attempt to use earlier

versions with this release.

XICE68K 6.00 is compatible with Microtec Research's MCC68K 4.1 or above

for ANSI support, MCC68K 3.3K or above for K&R support, ASM68K 6.7 or

above, and CCC68K 1.0A or above.

XICE version 6.00 requires that the emulator be running version 18.0 of

the shell code, and version 11.00 of the pod code.  These numbers may be

verified via the HWCONFIG command in XICE.  If they are incorrect, please

exit XICE and start up again with the '-e boot' option, which will

reload the software.  Failure to do this could result in erratic operation.

==============================

KNOWN DIFFERENCES IN OPERATION

==============================

This section describes items that have changed since the Software

Installation Guide, the XRAY Documentation Set, and the XICE Supplement

were produced, or operate differently from what is described in these

manuals.

1.  STARTUP ERRORS. If certain errors occur while XICE is starting up,

XICE may not pause between displaying the message and closing the

screen.

2.  WINDOW NUMBER. The 'vopen' command when 'option radix=hex' has been

executed will cause the window to open with the window number

interpreted as hexadecimal but displayed as decimal.  The coordinates

are also interpreted as hexadecimal.

3.  DATA VIEWPORT.  The data viewports 3 and 12 truncate symbol names at

11 characters.

4.  ACCESS BREAKPOINTS. Access breakpoints cannot be set on logical

addresses.

5.  TRACE DISASSEMBLER. The trace disassembler, when unable to find

any instructions, will sometimes display a "Valid Disassembly"

error message in addition to a message about being unable to find

any instructions.

6.  PRINTSYMBOLS. When displaying more than one page of symbols with

the PRINTSYMBOLS command, it is often necessary to hit the space

bar more than once to advance to the next page.

7.  CHECKING LOGICAL ADDRESSES.  XICE will hang if you attempt to do a

CRC over an address range that contains an invalid logical address.

8.  PERFORMANCE ANALYSIS.  If the target processor executes a STOP

instruction while performance analysis is being utilized, XICE

will crash.

9.  PERFINT.  The help for the PERFINT command states that values between

0 and 120 may be used.  In fact, the values are 1 to 120.

10. EVENT SYSTEM.  The event status mnemonic DSACK0 is not accepted in

event statements.  The values currently accepted are DSACK1, and

DSACK2.

11. TRACE COMMAND.  The SW_TRACE option is not accepted in the xice.cfg

file.

12. PERFFORMAT COMMAND.  The PERFFORMAT command does not work with the

PERCENT option.

13. EVENT SYSTEM.  Event system statements that are OR'd together will

not always trigger an action.

14. EVENT SYSTEM.  Setting an event with a counter not equal to 0 will

never trigger.

15. EVENT SYSTEM.  Events specified with a data comparator that is not

equal to a value will never trigger.

16. MOUSE BUTTONS.  At times the buttons on the top of the XICE window

will output garbage when selected.

17. CURSOR.  At times the cursor will disappear.

18. ACCESS BREAKS.  XICE is unable to determine that an access break

has occurred if trace has been shut off with the TRCBEG OFF

command.

19. It is not recommended that the HOST command be used, instead

open another window using your window manager.

20. A new command, LOADVEC, was added.  This command loads the CPU

restart vectors.

====================================================

PROBLEMS FIXED/ENHANCEMENTS MADE BY THE 6.00 RELEASE

====================================================

1.  X-windows support has been added.  This version will now determine

the environment being run, and start an X-windows or Sunview version

accordingly.

2.  Added support for C++.

3.  Added support for the 68ec030.

4.  Added statistical performance analysis.

5.  Added ability to initialize registers from the file xice.cfg.

Other major changes were made to the format of the file xice.cfg.

The XICE section has been removed, and the desired emulator is

now specified solely by the AMCEMUL environment variable.

6.  Added version checking of .pod, .shl, and .dnl files. The version

information resides in the new file release.dat. If this file is

not present or version numbers do not match, a warning is displayed.

7.  When starting up, XICE displays information on the emulator to which

it is attempting to connect. The HWCONFIG command displays the same

information about the emulator to which XICE is connected.

8.  Added TRCWINDOW command to allow viewing of trace capture windows

other than the first.

9.  Added BITRANS switch to allow execution breakpoints on logical

addresses.

10.  Added BPSPACE command to control which address spaces execution

breakpoints appear in.

11.  Added RUN_TIME command, which allows user to specify the number of

seconds to allow the target to stay in RUN.

12.  Increased TRCFRAME limit to 16K.

13.  Events set in multiple groups can be cleared properly.

14.  Trace-related commands that request trace greater than what is

captured now display the correct maximum trace frame number.

15.  FILL now reports verify errors when they occur, and reports the

offending address and data.

16.  If an error is encountered while starting up a diagnostic, it is

now reported, and if it is only a warning the diagnostic will

continue.

17.  The command 'TEST/R range' can now be stopped with ^C.

18.  The LOAD command now displays the sections being loaded, and the

addresses those sections occupy in target memory.

19.  Support has been added to specify access size, memory space,

and logical/physical attributes for all reads and writes of memory,

using the SIZE, SPACE, and ADDRESS commands.

20.  Changing RUN_POLL to a lower number no longer causes XICE to consume

all CPU time.

21.  The COMPARE command with the '/R' argument now stops at the end of

the compare range.

22.  Fixed multiple bugs in trace caching, so that changes in the contents

of the trace buffer are now detected.

23.  The FILL command now reports verify errors when they occur.

24.  Intermittent failures no longer occur when using the '-e boot' switch.

25.  Two DT commands entered in a row will now display the correct source

line both times.

26.  Added the command RUN_TIME, which allows the user to specify the

maximum number of seconds to allow the target to stay in RUN.

26.  XICE no longer reports incorrect access breakpoint addresses.

27.  The event system no longer intermittently crashes when the user tries

to set an event that is already used in an armed trigger.

28.  Handling of the A7 (stack pointer) register in user versus supervisor

mode has been fixed.